Notes on Draping Gowns

[image: image1.jpg]



By Mara Riley

Copyright January 2004
This paper may not be copied and distributed without the express permission of the author.

Introduction

18th Century gowns are always worn over stays, which give one’s figure the correct “ice cream cone” shape.  If you are draping your gown on a dressmaker’s dummy, or having a friend drape it to you, make sure you (or your dress form) are wearing your stays for the fittings.  A 19th century corset gives an “hourglass” rather than “ice cream cone” shape, and should not be used as the foundation for draping 18th century clothing.

Styles of gowns:

The 18th century gown grew out of the 17th century mantua, a sort of loose robe which was worn over a boned bodice and petticoat.  Eventually these robes were made more fitted by pleating (held in place by a belt), then the pleating was stitched down, and eventually the form-fitting 18th century gown emerged.

The French referred to the style of gown with the pleats stitched down in back from the neckline to the waist as the robe en fourreau (which means ‘in funnels’), or ‘robe a l’ Anglaise’, which probably refers to the origin of this style in England; indeed, one doesn’t see this style in French illustrations until about the 1760s.  The English themselves called this style a ‘nightgown’, which refers to its origin in the mantua, but doesn’t refer to how it was worn – this is the most commonly worn style for common English women’s everyday clothing.

The style of gown which has the pleats hanging loose from the shoulders (robe a la Française) was called the ‘sack’ or ‘saque’ by the English, or sometimes ‘negligee’.  Despite the term implying a state of undress, by the mid-18th century this style is generally considered the ‘dressier’ style.

From the beginning to the middle of the 18th century, gowns almost always have robings (descendants of the pleats of the original mantua) and stomachers, which might or might not match the fabric of the gown.  Around 1760 one starts to see gowns that fasten edge-to-edge in front, omitting the robings and stomacher.  The old style of gown, however, continues to be seen.

Another change around the 1760s is the move from cuffs on the sleeves of gowns to hanging ruffles, usually pinked.  The ‘round gown’ (closed in front, rather than split so that one can see the petticoat underneath) also becomes less popular in the latter part of the century.  The gown which reenactors call the ‘robe polonaise’ with the bodice cut entirely separately from the petticoats, comes into fashion somewhere between 1775-1780.  That term isn’t actually correct; we don’t know exactly what the period term for that style of gown was, though it might have been called a ‘fitted back gown’. The term ‘polonaise’ actually refers to the style of looping the petticoat up with ties, a style that came into vogue around 1772.

Fabrics: Silk (taffeta is a good choice; if one can find a period-looking damask or brocade, those will work too); linen, cotton (block-printed cottons were especially popular) wool, or linsey-woolsey.  


Thread: If using silk, use silk thread in a color as close to matching your fabric as possible.  If the gown is linen, use white or natural linen thread.  If using wool, either linen or silk thread can be used.  For cotton fabric, cotton hand-quilting thread is recommended.
Draping the lining

Unlike modern dresses, the inside lining of the gown is constructed first, using strong linen, either bleached or natural (greige).  The gown is then built, or ‘draped,’ onto the lining.

Pin the fabric to the stays and cut it to the proper shape (see books in bibliography for various styles); pencil in the outlines of the bodice lining with tailor’s chalk or a lead pencil.  You should leave a little extra fabric around the edges of each piece so you can adjust them as necessary.  

When the pieces are roughed out, unpin them and make sure the pieces are the same shape (i.e., mirror images of each other) for the right and left sides, with adjustments as necessary for the figure anomalies that most of us have, such as having one shoulder or one hip lower than the other.

The lining is in four pieces, two in front and two in back. Note that the center back seam curves in at the small of the back, and that the side seams are actually at the side-back, not the exact side, of the torso.  The side seams form a gently curving “C” shape, placing the emphasis toward the waistline. 

The back waistline should follow a pleasing curve angled from the down toward the center pleats, ending around the tailbone (though this may vary from figure to figure, as needed for a pleasing line).  The front waistline should drop from the side waistline to a curved point at the center front.  The bottom of the stays can often be used as a guide for these lines.

Gowns in the middle of the century have a long waist, a little below the natural waistline; waistlines then follow a rising trend until the end of the century.

The armscye (arm hole) comes in further than one would think – the sleeve cap usually comes to the inside of the shoulder bone at the top of the shoulder (feel for the bump at the top of the shoulder, and look at cutting diagrams from Waugh, Baumgarten and other sources).

Late 18th c. gowns may have some boning; if the back of the gown is boned, then the back of the stays can be lightly boned.  The boning is inserted into the bodice lining so that the casings are between the lining and the outside material, not inserted into separate casings on the interior of the bodice (a 19th c. technique).

[image: image2.jpg]


 [image: image3.jpg]


  [image: image4.jpg]



Draping the back pleats

Sew and press the back, shoulder and side-back seams of the lining. Baste the back center panel of the outer fabric to the bodice lining along the center line or crease.  Make sure the center of the lining and the center panel of fabric are lined up before basting, and smooth out any wrinkles.   If desired, you can make the first set of pleats, which run straight up and down the center back, at this point.  The pleats should be fairly shallow, usually about ¼” to ½”, though this will vary depending on the period and style.

Put the lining onto the figure being draped.  Arrange the pleats so that they are appropriate for the style being made* and for the figure being draped and pin into place.  Removing the lining from the figure, press the pleats, smoothing out any wrinkles, then re-pin, test-fit, and baste into place.  Stitch the pleats into place using a running stitch or back stitch placed every ¼” or so, with only a tiny bit of the stitch showing on the top of the fold.  You should stitch through all layers of fabric, including the lining.

*Pleats are wider at the beginning of the century, getting narrower toward the end.  The center back pleat is a box pleat, with the edges pointing inward; then there are two pleats pointing outward.

[image: image5.jpg]


  [image: image6.jpg]



Cut the extra material of the back on either side of the pleated section at an angle from the edge of the material (at the natural waistline) to the edge of the pleats (a little below the waistline) and baste to the lining.

[image: image7.jpg]



Cutting the skirt fabric away (bodice is at left)

[image: image8.jpg]



Cutting the back skirt fabric away from the gown’s bodice

[image: image9.jpg]



Turn the bottom edges of the bodice back outer fabric and lining in towards each other, and sew (use whipstitch or slipstitch)

[image: image10.jpg]oy





In this picture, you can see where the gown’s back skirts have been cut away from the back of the bodice and pleated behind the center back pleats.

There are two double-pleats (see finger and thumb) on each side, which serves to tuck the raw edge of the skirt fabric behind the gown bodice.

Draping the front of the bodice

Baste the front bodice (you can rough out the shape based on the lining shape) to the lining along the neckline, armholes, bottom, and side/back seams.  You might want to do a final fitting at this point, especially if you’re draping the gown on a dress form, adjusting the fit along the side-back seams.  Lap the front of the bodice over the back bodice at the side seams and whipstitch or backstitch in place.  

[image: image11.jpg]


  [image: image12.jpg]


 [image: image13.jpg]



Robings:

If you are making a gown that has robings in front, arrange and lightly stitch the front bodice pleats into position onto the lining (see cutting diagrams in Norah Waugh’s Cut of Women’s Clothes and other sources).  When you’ve got them pleated to your satisfaction, stitch them in place using a running or backstitch.  Sometimes the stitching is done underneath the robings so it doesn’t show; sometimes the robings are topstitched so that the stitches show.   You can also cut the robings separately as a tube, stitching them down to hide the diagonal edge of the bodice. 

Once the sleeves are sewn on (see below) and the front robings are arranged and sewn over the sleeve tops, the back neckline is covered by a narrow strip of material placed across the pleating and then mitered at the ends.

You can have either one or two robing folds on the front bodice, two being more common.  The width of the robings varies, depending on the proportions needed for the gown – a larger figure requires larger robings to look correct, and vice versa.

There is one gown in Bradfield’s ​Costume in Detail (pp. 3-4) which appears to be constructed with the robings separate from the bodice front.

Contrasting robings and cuffs seem to be a middle and lower class fashion.  They are not usually seen in upper class portraits.  A wool gown might have calico (i.e, block-printed printed cotton), checked linen, or silk robings and cuffs.

Flounces start appearing around the middle of the century: “Flounces were pleated, not gathered.  The edges of flounces made from the early stiff silks have a narrow hem and are edged with braid; later silks have these edges pinked.”  (Waugh, p. 76) 

Sleeves

Sleeves were usually cut on the horizontal grain until the 1780s.  This means that stripes go around the arm, not down it.

When draping the sleeve, note how far in the top of the sleeve is placed on the shoulder (see above notes on draping the torso) – it often comes to the inside of the shoulder, so feel for the bump marking that spot and look at cutting diagrams in Waugh, Baumgarten or other sources for comparison.

Drape the sleeve and use it as a pattern for the lining.  Sew sleeve seams along back of sleeve, place lining in sleeve, wrong sides together, and attach as one to bodice.

Sew the sleeve to the armscye (arm hole) from the inside under the arm as far as the head of the sleeve using backstitch (for strength).  If your gown has robings, leave the head (top) of the sleeve raw, and arrange it in small pleats onto the outside of the bodice at the top of the shoulders.  The raw edges of the sleeve are then covered by the front robings, which come over the shoulder to join with the outside back pleat to give the illusion of one continuous pleat coming over the shoulder.  

[image: image14.jpg]



[image: image15.jpg]




[image: image16.jpg]



[image: image17.jpg]




Draping the sleeve: lining fabric is pinned to bodice and along underarm

[image: image18.jpg]



[image: image19.jpg]




Sleeve lining is used to draft sleeve pattern.  (Note tuck drawn on paper pattern at right.)

If the gown has no robings, place the sleeve cap under the folded edge of the shoulder strap and slip-stitch in place from the right side, or sew the sleeves in from the inside all the way around. 

Finish the sleeve by turning the raw edges of the sleeve and lining toward each other and slipstitching together.  Attach cuffs according to their style. 

[image: image20.jpg]



Cuffs

Cuffs from the beginning to the middle of the century were usually some kind of wide cuff with one or more horizontal pleats or a winged cuff (hanging cuff, narrow at the top, hanging a few inches below the arm, attached only at the inside of the elbow and unattached at the back).  A hanging ruffle comes into vogue toward the middle of the century and remains popular until about 1775, when other more streamlined cuff treatments come into vogue.  Longer sleeves start to be seen around 1780.  Sometimes hanging cuffs contained a lead weight to give them the proper drape.

Skirts (the bottom half of the gown)

Skirts can either be closed (called a ‘round gown’) or open in front. Closed robes were less common in the second half of the century. The closed robe has the waistline of the front skirt panel pleated to a waistband to which tapes (ties) are attached; these are then tied behind the back underneath the gown, as one would tie on an apron. Then the back part of the gown is put on, and laced or pinned shut. Open-fronted gowns were often worn over matching petticoats, though non-matching petticoats seem to have been common as well.  Quilted petticoats were another popular option.

[image: image21.jpg]


 Pleats, back[image: image22.jpg]


Pleats, front

[image: image23.jpg]


 [image: image24.jpg]



Inside of skirt pleats, showing stay stitching

The circumference of petticoats on “common” gowns (as opposed to those with particularly wide hoops) should be about four times the waist measurement of the wearer.  Add panels of fabric to get the appropriate width.  If you are using 60” wide fabric, you will already have 60” for the back of the gown, and need to add only a 60” wide panel (in one piece or split down the middle, depending on the style of your gown).

Pleats on earlier gowns, which were made of stiffer silks, tend to be wide – about 1”.   As thinner silks come into vogue (1780s), pleats get narrower. Pleats usually point from the front toward the center back.

The skirts of gowns are not lined.  When the gown opens in front, the front edges are cut with an extra four to six inches, which are turned back to the inside of the gown to make facings.  

For the robe a l’ Anglaise, please note that the first two pleats are fairly deep pleats pointing outwards, hiding the raw edge of the skirt material where it was cut away from the bodice underneath the en fourreau (center back) pleats.

Arrange the front and back petticoats into pleats, stay-stitch, then pin to the bodice.  Adjust the petticoats so that the hem is level with the floor all the way around.  Turn the top edge of the petticoats under, and stitch (at the fold) to the bodice.  The edges of the petticoats ARE NOT sandwiched between the lining and the bodice outer fabric, as in modern clothing; rather they hang down inside the petticoats; when you look at the inside of 18th century gowns you can see the raw edges of the petticoats hanging down inside.  This is contrary to how modern seamstresses think of construction, but that’s how they did it.

Sew the side seams of the skirts together, leaving about 7” to 8” open at the top for pocket openings.

Make the skirts about ankle length then hem them along the straight grain.  The hem is not adjusted at the bottom; rather, it is adjusted along the top edge during pinning to the bodice.

Put the gown on the dress form or figure being draped and arrange the skirt in pleats (over hip pads, a bumroll, or hoops, if worn) and pin them to the bodice so that the hem is an even distance from the floor.  If one is using a bumroll or hoops, this means that the skirt should be a bit longer at the sides where it comes over the bumroll or hoops than at the back and front.  

Once the skirts are pleated, then the raw edge on top is turned under (1”-2”) and whip-stitched to the bodice.

Hems

Several treatments can be used. 

· narrowly turned under (usually not more than 1”, often less)

· faced with a strip of silk or other fabric (such as linen) 4-12” wide; this may be useful for gowns used for dancing, as the facing keeps the wearer from catching a foot in the hem

· turned up, and wool tape applied with a running stitch along both edges of the tape

Fastenings

Most gowns seem to have been pinned or basted on, or sometimes laced.  If the gown has lacing holes, the holes are usually let into the bodice lining and are concealed behind the gown’s robings (in the example on pp. 17-18 in Bradfield, the robings are applied separately).  You can also attach a separate lacing strip to the inside of the gown’s bodice.

If wearing a gown with robings, pin the stomacher to your stays, then pin or baste the gown to the stomacher, or lace the gown over the stomacher.

If your gown fastens edge-to-edge, you can either baste yourself into it or use straight pins to pin the gown shut.  Some reenactors use a hook-and-eye fastening in the lining, then baste or pin the outer layer of the gown together.

Stomachers

On gowns with stomachers, the stomacher is a triangular shape big enough to fill the opening at the front of the gown.  Many stomachers have little ‘wings’ at the bottom of the stomacher, which may have served to provide additional coverage if the bottom edge of the gown’s bodice flipped open.  Stomachers could match the fabric of the gown itself, or embroidered with crewelwork or trapunto.  The stomacher was pinned to the stays through tabs attached to the sides of the stomacher; the gown was then laced or pinned on over the stomacher.

Trim

A plain (everyday) gown doesn’t require much trimming; a simple cuff or self-fabric ruffle may be all that is needed, and even that is optional.  Rectangular or trapezoidal cuffs and robings were usually of self-fabric, but were occasionally of a different color and/or fabric from the body of the gown.

For fancy dress, some simple additions include a “breast knot”, a bit of ribbon tied in a bow and pinned to the breast of the gown.  If the gown’s sleeve has a self-fabric ruffle at the cuff, you can baste an ‘engageante’ (muslin, lace or lace-edged muslin) to the inside of the ruffle.  Engageantes are often mistaken for a ruffle on the end of a shift sleeve, but in fact were usually separate pieces that, being more delicate, were removed and washed separately from the shift.

----------------------------

Sources:

Barnwell, Susan and Holford, Mary: Pattern Diagrams for Three Eighteenth-Century Dresses in the Royal Ontario Museum

Baumgarten, Linda, Eighteenth Century Clothing at Williamsburg

Baumgarten, Linda and John Watson with Florine Carr, Costume Close-Up: Clothing Construction and Pattern 1750-1790
Burnston, Sharon Ann, Fitting & Proper: 18th Century Clothing from the Collection of the Chester County Historical Society
Cunnington, C. Willett and Phillis, The History of Underclothes
Waugh, N., The Cut of Women’s Clothes
[image: image25.jpg]



