Stays
Step By Step Instructions for the Easily Intimidated

[image: image1.png]



by Mara Riley

and Kate Johnson

Dedicated to Drea Leed for getting me started with costume drafting, and to Kass McGann, who shares the obsession.
Thanks also to Barbara Smith for pointing out steps I forgot to include because I’ve been doing this so long!

Introductiontc "Introduction"
Stays are one of the most basic components of period costume, yet many people find them quite intimidating to make and to wear -- the former because they seem complicated at first glance, the latter because many ready-made stays are uncomfortable, unless they happen by sheer luck to fit your body type.
A well-fitted set of middle-class 18th century stays should be snug, but should not be uncomfortable or greatly limit movement.  After all, most women in the 18th century had to do housework and farmwork, and did so wearing stays.  Comparisons between upper-class stays and those of middle-class women show this difference -- the stays of women who lived a more active life have wider armscyes and are less likely to have widely-spaced straps to hold the shoulders back in an extreme posture.  Stays should provide good back support, much like the modern back-braces worn by people whose jobs involve lifting.  When lifting an iron pot off the fire, you may appreciate the additional support!

Another common misconception is that stays were ‘underwear’.  Well, not quite -- the best modern equivalent would be a sports bra.  You might wear your sports bra around the house, doing laundry, mowing the lawn, and so on, but wouldn’t dream of going to the grocery store or church without putting something over it.  Women in the 18th century wore their stays in similar situations -- picking apples, doing laundry or other chores.

There were professional staymakers in the 18th century, and their work is more likely to have survived the centuries, because fancier clothing is often saved where the plain is used up and thrown out. However, some women apparently made their own stays at home, judging from a few surviving examples.  

Given how few professional staymakers there are today, knowing how to design and fit your own stays is a good skill to have.  You may even wind up making them for friends.  In addition, you’ll learn the relationship between body measurements and the drafted pattern and can use these skills to go further in learning to draft and drape period costumes.

Lastly, if this is your first pair of stays, you might want to make them in an inexpensive heavy cotton fabric before cutting into your good linen; wear this ‘starter set’ for a year and then make your adjustments to the final set, lengthening the waist or changing tab placement as necessary.

Good luck, and happy sewing!

Materials Needed:tc "Materials Needed\:"
Two sheets of regular posterboard
Linen or hemp canvas for interior of stays (7 oz. or heavier, tight weave) -- 1-1/2 to 2 yards of 60” wide fabric, prewashed to minimize shrinkage.

Lightweight outer fabric -- linen or lightweight wool, or, for upper-class stays, a period-appropriate silk damask or brocade -- 1 yard of 60” wide fabric, prewashed

Lightweight linen lining fabric -- 1 yard of 60” wide fabric, prewashed

white kidskin leather (most commonly used binding in the 18th century) 

OR

1” strips of linen fabric, cut on the straight of the grain (NOT the bias - bias binding is 19th century) OR

10 feet 1” wide linen tape (ribbon), prewashed OR 

1 sheet of white/beige (undyed) automotive chamois leather, cut into 1” strips

Boning: 3/16” or 1/4” half-oval basket caning, reed bents, German plastic boning (mimics whalebone (can be obtained from suppliers listed in back), or 1/4” steel boning (should be ordered after you have your pattern drafted and figure out the lengths needed). If you use basket caning, you may want to use some steel along the front and/or rear lacing edges for added strength.

Carpenter’s awl (available at hardware stores)

Architect’s square (available at office supply stores)

Architect’s ruler (available at office supply stores)

White or natural linen thread and/or cotton quilter’s thread (the latter is easier to use in a sewing machine)

Beeswax, for waxing linen thread for handsewing
Pencil, needles, pins

Corset busk (a 12”  wooden ruler or paint stick from the hardware store can work in a pinch)

Drafting the Basic Shapetc " Drafting the Basic Shape"
This portion of the stays instructions is borrowed with permission from Drea Leed’s Elizabethan Corsetry web site:

http://costume.dm.net/custompat/
with modifications made for  18th c. corsets. 

If you want to skip most of the calculations below done for you online, just visit her site and plug in your numbers.

Measurements:

Waist = ____ 
divided by 2 = 


____


Bust = ____ minus 2 inches, divided by 2 = 
____


Waist-to-underarm = 



____


Front length of stays = 


____


Bra cup size =

 


____

Drafting the Top Line:
Using a piece of posterboard, mark as follows:

On the right edge of the cardboard, mark the front length measurement.

Along the top of the posterboard, mark the bust measurement (minus two inches, then divided in half).  The reason for subtracting two inches is so that you will have a two-inch gap when you lace the stays.  If the edges of your stays meet, they won’t be snug enough to give you a good fit.

Find the middle of the bust line.  Measure two inches to the left, and measure down:


1 inch if you are a size A or B cup


2 inches if you are a size C cup


3 inches if you are a size D cup or larger

Mark this as Point A.

Take this same measurement of one, two or three inches, go to the left end of your horizontal bust line, and measure down from there the same number of inches.  Label this Point B.

Measure from the center front 1/4 the length of the bust line, and mark this Point C.

Draw a gently curving line to connect points A, B and C.  This is the top profile of your stays.

Drafting the Waistline:
From Point A, measure down the length of your underarm-to-waist measurement.  From this point, draw a horizontal line all the way across the paper.

Subtract two inches from your waist measurement, and divide it in half.  This will give you the waistline length for your stays.  Measure out this far to the left along the waistline, mark, and measure down one inch.  Label this as Point D. Connect D and B to form the back center of your corset.  

Divide the waistline in fourths.  Measure over one-fourth on the waistline, and mark this Point E.  Measure over another fourth and up one inch; mark this as Point F.  Measure over another fourth and up half an inch; mark this Point G.

Draw a curving line from the bottom front center of the corset to point G.   Make sure the curve at the bottom is wide enough to fit the point of the busk. 

 Measure up from Point E one inch, and mark it. Draw a gentle curve from Points G, to this mark, back down to point F, and then from F to D. This finishes the body of the corset. 

Straps:
If you want a corset with straps, Measure in three inches from the back center, mark it, measure in one more inch, and draw a set of 10 inch long parallel lines 1” apart, perpendicular to the top curve, to form the strap.  You may want to add the strap to your stays pattern when you do the fitting so you can  adjust the angle at which the strap attaches to the stays in front and back; in this case, make a strap about 10” long and 1” to 1-1/2” wide, and tape it to the stays in back about 3” from the center; when fitting the stays, bring this over your shoulder and mark where it meets the front of the stays.

(add illustration)
Fitting the Pattern:
Using this half-pattern, cut another matching piece of posterboard, tape it (using strong tape like duct tape or medical tape) together at the center front, and ‘try it on’ for fit.  If you don’t have a helper to tape it at the back, tape it at the center back with a 2” gap, then put it on, taping the center front together.  Your stays should not close completely in the back; there should be a one- to two-inch gap, some of which will disappear when you lace on the actual corset.

Make sure that the underarm curves and hip curves are large enough so that they won’t rub against your arms and hips. You may have to move the underarm/hip curve slightly to the front or back to get a perfect fit. 

Fit your straps at this stage; most 18th c. stays with straps appear to have the straps tied at the front.

Dividing the Pattern and Adding Tabs:
It helps to have an architect’s scale ruler (a three-sided ruler available at most office supply stores) to draw in the boning channel lines.  One of the edges of the ruler has a scale marked for lines every 3/8” inch.  This is the scale you use for drawing boning channels for 3/16” or 1/4” half-oval basket caning or for 1/4” steel boning.

To draw the boning channels, start at the center back line, and, using an architect’s square, draw two lines perpendicular to it.  Along these lines, mark off four channels (five lines) at intervals of 3/8”.  The first channel will have boning in it; if you are using basket caning, you may want to insert a piece of steel boning in between the caning to reinforce the center back line.  The second channel will have no boning; this is where your lacing eyelets go.  The third and fouth channels will be boned.

Continue drawing in boning channels to match the style of corset appropriate for your period; you will need to figure out how to divide up the pattern into the appropriate pieces (see illustrations), up to four or five pieces per side.  This keeps the fabric on the straight of the grain, and (in later styles) allows some curved shaping of the seams.  

Once the boning channels are sketched in, divide the stays pattern up into the appropriate number of pieces (just eyeball it -- this will vary depending on the style of the stays you’re making), cut the pattern apart, re-trace the pieces onto another piece of posterboard, and aseam allowances at the edge of each piece where they’ll be sewn together.  You don’t need to add seam allowances at the top and bottom of the pieces. 

Most 18th c. stays had tabs at the bottom, which serve to keep the bottom edge of the stays from digging in at the waist.  If you wear your petticoats over your stays, the tabs also help keep the ties of the petticoats’ waistbands from digging in. If boning your stays with half-oval basket caning, you need to fully bone the corset and plan on at least three but usually four or five boning channels per tab, which allows the boning to reinforce itself.   If  boning your stays with steel  or plastic boning, tab width is less critical and the tabs can be narrower.  To draw the tabs, extend the bottom edge of each piece by 2-1/2”  or so and divide the extension into tabs as appropriate for the style of corset you’re making. Cut between the tabs on the pattern so you can draw them onto the fabric, but don’t cut between the tabs on the fabric until after you’ve sewn the boning channels in.

If you want stays that lace over a stomacher, you can leave the center front of the stays separate from the rest of the stays;  add about one channel’s width to the stomacher so that the edges of the stays and stomacher overlap when they’re laced up. 

(Need illustration of various styles of stomachers)
Cutting Out, Sewing and Assembly

Lay the pattern pieces on your heavyweight linen interior fabric (two layers) on the straight grain of the material.  If the material’s grain isn’t straight, tug it on opposite corners until the warp and weft until it is. Trace around the posterboard with a pencil, marking the linen.  Pin the two linen layers together, and cut out the individual pieces. Mark the  boning channels onto the fabric, as you have them marked on your pattern.

Pin  the interior linen pieces onto your outer fabric, on the straight grain of the material. Rough cut around each piece (leave at least 1/2” of material).  Sew the interior pieces onto the outer fabric, including all the boning channels, using cotton quilter’s thread.  Leave one end of the boning channels open so you can insert the boning. You may need to adjust the tension of your sewing machine to accomodate all the layers; try this out on a scrap of fabric before starting to sew the stays.

When the boning channels are all sewn, assemble the pieces of the corset and sew them together.  Iron the seam allowances down.  After you insert the boning, trim the outer fabric and tack the seam allowances down with a whip-stitch.

Some period stays have the boning channel stitching hidden, though most have the channels visible. If you want the boning channels to be invisible from the outside,  sew the boning channels into the interior canvas and then apply the outer fabric.

Alternate method: some 18th c. stays may have had the boning sewn in channel by channel; if you want to do this, sew the first seam in each piece, insert a piece of boning, then sew next to the boning using a zipper foot (or backstitch, if hand-sewing).  You can use either method if you are sewing by hand, using backstitch. If you do it this way, it will be more difficult to sew the pieces of the stays together, so leave a wider seam allowance and use a zipper foot to get the sewing close to the boning channels.  You’ll sew the seam over (or close to) the outermost boning channel seam. If sewing by hand, use backsttich. Some period stays also have a strip of leather or narrow linen tape covering the seam on the outside, sewn down with a running stitch, as a decorative detail.

Inserting the Boning:
If you are using steel  boning, measure each boning channel and allow 

1/4” at each end for ‘wiggle room’, then order your boning  If you are using half-oval basket caning or reed bents, insert the  boning in the channel, mark the end of the channel with a pencil, pull the boning out of the channel a little bit and cut the boning about 1/4” short of the pencil line and push it back into the channel.  Do the same with a second piece of caning, inserted flat-sides-together with the first piece.  Move on to the next channel until you have all the boning inserted.

At this point you can either sew the lining fabric (cut using the same pattern pieces) , or you can sew it on after you apply the binding.  Lining fabric was often sewn on after the binding,  so that it could be replaced (or, frequently, patched) as it wore out. 

Applying the Binding:
This is possibly the most tedious and annoying part of making stays.  To sew on the binding, pin it to the edge of the stays as shown in the picture -- edges lining up -- and sew.  A zipper foot may help. Fold the binding over the edge and whip-stitch to the back of the stays.  You might want to sew the boning onto the tabs by hand, as getting it fitted into the tight corners using a sewing machine can be difficult.

Period method: Sewing the binding on by hand, use a running stitch to sew the binding to the front of the stays (i.e., to the front of each boning channel, not through all the layers) then fold the binding over the edge of the stay to the back, and whip-stitch.  The leather binding becomes the ‘cap’ for the boning channel, holding the boning in. The tabs can be bound separately with wider strips of leather, so that the entire back of the tab is backed with leather.

If you are using strips of linen from the stays fabric, cut them on the straight of the grain, not the bias (bias strips date from the 19th century).  You can also use the linen bookbinder’s tape (ribbon) available from various 18th century sutlers (merchants).

Many 18th c. stays were bound in kidskin leather.  This is hard to find today, but some sutlers carry it.  An easily obtainable alternative is chamois leather, of the type carried in automotive stores for washing cars; cut this into 1” strips and use for binding. A  friend tells me that even though she got light-colored chamois, it left yellow stains on her chemise where it got damp, so prewashing the chamois might be a good idea.  Dampen the leather before sewing it; this makes it easier for the needle to pass through.  Also, you can use a leather needle (available at fabric stores), though you should be careful not to cut your fabric as you sew.  A thimble or sailmaker’s palm will be very helpful, as will a small strip of leather for grasping the needle and pulling it through.

It may be easier to sew the binding onto the bottom tabs by hand, if you can’t get the sewing machine into the tight angles.

Making the Eyelets:
To make the eyelets for lacing your stays, mark the holes so that the stays can be laced in a spiral pattern.  Space the holes about 1-1/2” apart; the final number of holes will depend on the size of your stays. Note that your first two holes on one side will be close together  to give proper spacing to the rest of the holes. 

(need illustration of spiral lacing)
Take the carpenter’s awl, or other similar object (some people use knitting needles) and poke it through all the layers of fabric.  You want to push the fibers apart as much as possible, though some threads will break.  Pushing the fibers apart with an awl instead of punching a hole gives you a stronger eyelet that is less likely to deform with use. Sew the resulting hole open using whip stitches -- about 15 to 20 stitches per eyelet should be fine.  The hole should be big enough to get your lacing cord and tip through, about 1/8” to 3/16” wide.

Once your eyelets are in, take your lacing cord, seal the end. If you have a metal point of the right size, use that, but otherwise use either wax, glue, or fingernail polish. You can also wrap thread evenly around the end of the cord, making a neat tip, then apply the glue to keep the thread in place. Tie the cord off to the bottom of your stays, and lace them up.  Allow enough extra cord to tie off at the top, trim, and seal the other end.


That’s it!

[image: image2.png]



This portion of the stays instructions is borrowed with permission from Drea Leed’s Elizabethan Corsetry web site:

http://costume.dm.net/custompat/
with modifications made for  18th c. corsets. 

If you want to skip most of the calculations below done for you online, just visit her site and plug in your numbers.

Measurements:

Waist = ____ 
divided by 2 = 


____


Bust = ____ minus 2 inches, divided by 2 = 
____


Waist-to-underarm = 



____


Front length of stays = 


____


Bra cup size =

 


____

Drafting the Top Line:
Using a piece of posterboard, mark as follows:

On the right edge of the cardboard, mark the front length measurement.

Along the top of the posterboard, mark the bust measurement (minus two inches, then divided in half).  The reason for subtracting two inches is so that you will have a two-inch gap when you lace the stays.  If the edges of your stays meet, they won’t be snug enough to give you a good fit.

Find the middle of the bust line.  Measure two inches to the left, and measure down:


1 inch if you are a size A or B cup


2 inches if you are a size C cup


3 inches if you are a size D cup or larger

Mark this as Point A.

Take this same measurement of one, two or three inches, go to the left end of your horizontal bust line, and measure down from there the same number of inches.  Label this Point B.

Measure from the center front 1/4 the length of the bust line, and mark this Point C.

Draw a gently curving line to connect points A, B and C.  This is the top profile of your stays.

Drafting the Waistline:
From Point A, measure down the length of your underarm-to-waist measurement.  From this point, draw a horizontal line all the way across the paper.

Subtract two inches from your waist measurement, and divide it in half.  This will give you the waistline length for your stays.  Measure out this far to the left along the waistline, mark, and measure down one inch.  Label this as Point D. Connect D and B to form the back center of your corset.  

Divide the waistline in fourths.  Measure over one-fourth on the waistline, and mark this Point E.  Measure over another fourth and up one inch; mark this as Point F.  Measure over another fourth and up half an inch; mark this Point G.

Draw a curving line from the bottom front center of the corset to point G.   Make sure the curve at the bottom is wide enough to fit the point of the busk. 

 Measure up from Point E one inch, and mark it. Draw a gentle curve from Points G, to this mark, back down to point F, and then from F to D. This finishes the body of the corset. 

Straps:
If you want a corset with straps, Measure in three inches from the back center, mark it, measure in one more inch, and draw a set of 10 inch long parallel lines 1” apart, perpendicular to the top curve, to form the strap.  You may want to add the strap to your stays pattern when you do the fitting so you can  adjust the angle at which the strap attaches to the stays in front and back; in this case, make a strap about 10” long and 1” to 1-1/2” wide, and tape it to the stays in back about 3” from the center; when fitting the stays, bring this over your shoulder and mark where it meets the front of the stays.

(add illustration)
Fitting the Pattern:
Using this half-pattern, cut another matching piece of posterboard, tape it (using strong tape like duct tape or medical tape) together at the center front, and ‘try it on’ for fit.  If you don’t have a helper to tape it at the back, tape it at the center back with a 2” gap, then put it on, taping the center front together.  Your stays should not close completely in the back; there should be a one- to two-inch gap, some of which will disappear when you lace on the actual corset.

Make sure that the underarm curves and hip curves are large enough so that they won’t rub against your arms and hips. You may have to move the underarm/hip curve slightly to the front or back to get a perfect fit. 

Fit your straps at this stage; most 18th c. stays with straps appear to have the straps tied at the front.

Dividing the Pattern and Adding Tabs:
It helps to have an architect’s scale ruler (a three-sided ruler available at most office supply stores) to draw in the boning channel lines.  One of the edges of the ruler has a scale marked for lines every 3/8” inch.  This is the scale you use for drawing boning channels for 3/16” or 1/4” half-oval basket caning or for 1/4” steel boning.

To draw the boning channels, start at the center back line, and, using an architect’s square, draw two lines perpendicular to it.  Along these lines, mark off four channels (five lines) at intervals of 3/8”.  The first channel will have boning in it; if you are using basket caning, you may want to insert a piece of steel boning in between the caning to reinforce the center back line.  The second channel will have no boning; this is where your lacing eyelets go.  The third and fouth channels will be boned.

Continue drawing in boning channels to match the style of corset appropriate for your period; you will need to figure out how to divide up the pattern into the appropriate pieces (see illustrations), up to four or five pieces per side.  This keeps the fabric on the straight of the grain, and (in later styles) allows some curved shaping of the seams.  

Once the boning channels are sketched in, divide the stays pattern up into the appropriate number of pieces (just eyeball it -- this will vary depending on the style of the stays you’re making), cut the pattern apart, re-trace the pieces onto another piece of posterboard, and aseam allowances at the edge of each piece where they’ll be sewn together.  You don’t need to add seam allowances at the top and bottom of the pieces. 

Most 18th c. stays had tabs at the bottom, which serve to keep the bottom edge of the stays from digging in at the waist.  If you wear your petticoats over your stays, the tabs also help keep the ties of the petticoats’ waistbands from digging in. If boning your stays with half-oval basket caning, you need to fully bone the corset and plan on at least three but usually four or five boning channels per tab, which allows the boning to reinforce itself.   If  boning your stays with steel  or plastic boning, tab width is less critical and the tabs can be narrower.  To draw the tabs, extend the bottom edge of each piece by 2-1/2”  or so and divide the extension into tabs as appropriate for the style of corset you’re making. Cut between the tabs on the pattern so you can draw them onto the fabric, but don’t cut between the tabs on the fabric until after you’ve sewn the boning channels in.

If you want stays that lace over a stomacher, you can leave the center front of the stays separate from the rest of the stays;  add about one channel’s width to the stomacher so that the edges of the stays and stomacher overlap when they’re laced up. 

(Need illustration of various styles of stomachers)
Cutting Out, Sewing and Assembly

Lay the pattern pieces on your heavyweight linen interior fabric (two layers) on the straight grain of the material.  If the material’s grain isn’t straight, tug it on opposite corners until the warp and weft until it is. Trace around the posterboard with a pencil, marking the linen.  Pin the two linen layers together, and cut out the individual pieces. Mark the  boning channels onto the fabric, as you have them marked on your pattern.

Pin  the interior linen pieces onto your outer fabric, on the straight grain of the material. Rough cut around each piece (leave at least 1/2” of material).  Sew the interior pieces onto the outer fabric, including all the boning channels, using cotton quilter’s thread.  Leave one end of the boning channels open so you can insert the boning. You may need to adjust the tension of your sewing machine to accomodate all the layers; try this out on a scrap of fabric before starting to sew the stays.

When the boning channels are all sewn, assemble the pieces of the corset and sew them together.  Iron the seam allowances down.  After you insert the boning, trim the outer fabric and tack the seam allowances down with a whip-stitch.

Some period stays have the boning channel stitching hidden, though most have the channels visible. If you want the boning channels to be invisible from the outside,  sew the boning channels into the interior canvas and then apply the outer fabric.

Alternate method: some 18th c. stays may have had the boning sewn in channel by channel; if you want to do this, sew the first seam in each piece, insert a piece of boning, then sew next to the boning using a zipper foot (or backstitch, if hand-sewing).  You can use either method if you are sewing by hand, using backstitch. If you do it this way, it will be more difficult to sew the pieces of the stays together, so leave a wider seam allowance and use a zipper foot to get the sewing close to the boning channels.  You’ll sew the seam over (or close to) the outermost boning channel seam. If sewing by hand, use backsttich. Some period stays also have a strip of leather or narrow linen tape covering the seam on the outside, sewn down with a running stitch, as a decorative detail.

Inserting the Boning:
If you are using steel  boning, measure each boning channel and allow 

1/4” at each end for ‘wiggle room’, then order your boning  If you are using half-oval basket caning or reed bents, insert the  boning in the channel, mark the end of the channel with a pencil, pull the boning out of the channel a little bit and cut the boning about 1/4” short of the pencil line and push it back into the channel.  Do the same with a second piece of caning, inserted flat-sides-together with the first piece.  Move on to the next channel until you have all the boning inserted.

At this point you can either sew the lining fabric (cut using the same pattern pieces) , or you can sew it on after you apply the binding.  Lining fabric was often sewn on after the binding,  so that it could be replaced (or, frequently, patched) as it wore out. 

Applying the Binding:
This is possibly the most tedious and annoying part of making stays.  To sew on the binding, pin it to the edge of the stays as shown in the picture -- edges lining up -- and sew.  A zipper foot may help. Fold the binding over the edge and whip-stitch to the back of the stays.  You might want to sew the boning onto the tabs by hand, as getting it fitted into the tight corners using a sewing machine can be difficult.

Period method: Sewing the binding on by hand, use a running stitch to sew the binding to the front of the stays (i.e., to the front of each boning channel, not through all the layers) then fold the binding over the edge of the stay to the back, and whip-stitch.  The leather binding becomes the ‘cap’ for the boning channel, holding the boning in. The tabs can be bound separately with wider strips of leather, so that the entire back of the tab is backed with leather.

If you are using strips of linen from the stays fabric, cut them on the straight of the grain, not the bias (bias strips date from the 19th century).  You can also use the linen bookbinder’s tape (ribbon) available from various 18th century sutlers (merchants).

Many 18th c. stays were bound in kidskin leather.  This is hard to find today, but some sutlers carry it.  An easily obtainable alternative is chamois leather, of the type carried in automotive stores for washing cars; cut this into 1” strips and use for binding. A  friend tells me that even though she got light-colored chamois, it left yellow stains on her chemise where it got damp, so prewashing the chamois might be a good idea.  Dampen the leather before sewing it; this makes it easier for the needle to pass through.  Also, you can use a leather needle (available at fabric stores), though you should be careful not to cut your fabric as you sew.  A thimble or sailmaker’s palm will be very helpful, as will a small strip of leather for grasping the needle and pulling it through.

It may be easier to sew the binding onto the bottom tabs by hand, if you can’t get the sewing machine into the tight angles.

Making the Eyelets:
To make the eyelets for lacing your stays, mark the holes so that the stays can be laced in a spiral pattern.  Space the holes about 1-1/2” apart; the final number of holes will depend on the size of your stays. Note that your first two holes on one side will be close together  to give proper spacing to the rest of the holes. 

(need illustration of spiral lacing)
Take the carpenter’s awl, or other similar object (some people use knitting needles) and poke it through all the layers of fabric.  You want to push the fibers apart as much as possible, though some threads will break.  Pushing the fibers apart with an awl instead of punching a hole gives you a stronger eyelet that is less likely to deform with use. Sew the resulting hole open using whip stitches -- about 15 to 20 stitches per eyelet should be fine.  The hole should be big enough to get your lacing cord and tip through, about 1/8” to 3/16” wide.

Once your eyelets are in, take your lacing cord, seal the end. If you have a metal point of the right size, use that, but otherwise use either wax, glue, or fingernail polish. You can also wrap thread evenly around the end of the cord, making a neat tip, then apply the glue to keep the thread in place. Tie the cord off to the bottom of your stays, and lace them up.  Allow enough extra cord to tie off at the top, trim, and seal the other end.


That’s it!

[image: image3.png]



